### Appendix G.1 — ET2/ET5/ET6 Code

Appendix G.2 — ET3 Code

Appendix G.3 — ET4 Code

Appendix G.4 — ET7 Code

#include <stdio.h>

#include <string.h>

#include <stdlib.h>

#include <math.h>

#include "pico/stdlib.h"

#include "btstack_run_loop.h"

#include "hello_ble.h"

#include "pico/multicore.h"

#include "hardware/i2c.h"

// ── Tunable ───────────────────────────────────────────────────────────────────

#define MAX_EXERCISES      16

#define ACTIVE_REP_SEC      5    // seconds per active rep

#define MAX_DATA_ENTRIES  300    // session data log capacity

#define FLUSH_INTERVAL_MS  30    // ms between consecutive DATA| sends during flush

#define DIAG_PACKETS      25    // packets sent per DIAGTEST burst

#define DIAG_INTERVAL_MS 200    // 200 ms between packets = 5 Hz

// ── Motor (M1) — pins match KINETIX_test.c ───────────────────────────────────

#define MOTOR_PUL_PIN   0

#define MOTOR_DIR_PIN   1

#define MOTOR_ENA_PIN   2

#define MOTOR_DELAY_US  2500   // µs per half-pulse

#define MOTOR_DURATION  17000  // ms for one motor phase (FWD or BWD)

// ── HX711 load cell (H0 = plantar force) ─────────────────────────────────────

// Pin assignments verbatim from MOTORWORKS.c

#define H0_DATA   12

#define H0_CLOCK  13

// Calibration values from MOTORWORKS.c — re-run curr_mode==5 if hardware changes.

static int32_t hx711_offset[1] = { 1125691 };

static float   hx711_scale[1]  = { -800.7546f };

// ── I2C and IMU (ADXL345 via TCA9548A) ─────────────────────────────────────

// Pin assignments and channel mapping verbatim from KINETIX_test.c.

#define I2C_PORT      i2c0

#define I2C_SDA_PIN   4

#define I2C_SCL_PIN   5

#define I2C_BAUDRATE  100000

#define IMU1_CH       5   // thigh

#define IMU2_CH       6   // shank

#define IMU3_CH       7   // back / trunk

#define ADXL345_ADDR  0x53

#define TCA9548A_ADDR 0x70

#define MOTOR_POLL_MS   50     // how often Core 0 checks for motor completion

#define ROLLING_N        8     // rolling average window (samples)

// ── Phase ─────────────────────────────────────────────────────────────────────

typedef enum {

PHASE_IDLE,

PHASE_WARMUP_RX,

PHASE_WARMUP_RUN,

PHASE_SESSION_RX,

PHASE_SESSION_RUN,

PHASE_FLUSH_RUN,

} phase_t;

static phase_t phase = PHASE_IDLE;

// ── Exercise plan ─────────────────────────────────────────────────────────────

typedef struct {

char    code[20];

uint8_t sets;

uint8_t reps;

char    mode;    // 'A' active, 'P' passive

uint8_t param;   // active: resistance level   passive: seconds/rep

} exercise_t;

static uint8_t    plan_expected = 0;

static uint8_t    plan_received = 0;

static exercise_t plan[MAX_EXERCISES];

// ── Session data log ──────────────────────────────────────────────────────────

typedef struct {

char     code[20];

uint32_t t_ms;

uint8_t  rep;    // 1-based

uint8_t  set;    // 1-based

float    hip;

float    knee;

float    pf;

float    tf;

float    mf;

float    lf;

} data_entry_t;

static data_entry_t data_log[MAX_DATA_ENTRIES];

static uint16_t     data_count = 0;

// ── Session control ───────────────────────────────────────────────────────────

static volatile bool stopped     = false;

static volatile bool paused      = false;

static volatile bool timer_armed = false;

// ── Timer contexts ────────────────────────────────────────────────────────────

typedef struct {

btstack_timer_source_t timer;

uint8_t  ex_idx;

uint8_t  set_idx;

uint8_t  rep_idx;

uint8_t  tick;        // elapsed seconds within current rep

uint32_t total_tick;  // total ticks across session (for t_ms)

} run_ctx_t;

static run_ctx_t run_ctx;

typedef struct {

btstack_timer_source_t timer;

uint16_t idx;   // index of next entry to send

} flush_ctx_t;

static flush_ctx_t flush_ctx;

// ── Diagnostic context ────────────────────────────────────────────────────────

typedef struct {

btstack_timer_source_t timer;

uint8_t seq;   // current packet number (1–DIAG_PACKETS)

} diag_ctx_t;

static diag_ctx_t diag_ctx;

static bool       diag_running = false;

// =============================================================================

// Forward declarations — required because motor_session_poll_cb and arm_timer

// reference functions defined later in the file.

// =============================================================================

static float get_hip_angle(void);

static float get_knee_angle(void);

static float get_plantar_force(void);

static float get_thigh_force(void);

static float get_medial_force(void);

static float get_lateral_force(void);

static void  tick_callback(btstack_timer_source_t *ts);

// HX711 symbols and data used by core1_motor_entry (defined later in file)

static void  hx711_init(uint8_t data_pin, uint8_t clock_pin);

static float hx711_read_force_nowait(uint8_t data_pin, uint8_t clock_pin, uint8_t unit);

// Plantar force cache: written by Core 1, read by Core 0 via get_plantar_force().

static volatile float pf_latest = 0.0f;

// =============================================================================

// Motor — Core 1 runs the blocking step loop; Core 0 (BLE) is never blocked.

// Core 0 sets motor_req_pending, Core 1 clears it and sets motor_is_done.

// A btstack poll timer on Core 0 detects motor_is_done and sends MOTOR_DONE.

// volatile is sufficient here: single writer / single reader per flag.

// =============================================================================

static volatile bool     motor_req_pending = false;

static volatile bool     motor_req_fwd     = true;

static volatile uint32_t motor_req_ms      = 3000;

static volatile bool     motor_is_done     = false;

static volatile bool     motor_stop_req    = false;  // set by Core 0 to abort Core 1 mid-run

// ── stepper helpers (verbatim from KINETIX_test.c) ───────────────────────────

static void stepper_init_m(void) {

gpio_init(MOTOR_PUL_PIN); gpio_set_dir(MOTOR_PUL_PIN, GPIO_OUT);

gpio_init(MOTOR_DIR_PIN); gpio_set_dir(MOTOR_DIR_PIN, GPIO_OUT);

gpio_init(MOTOR_ENA_PIN); gpio_set_dir(MOTOR_ENA_PIN, GPIO_OUT);

gpio_put(MOTOR_ENA_PIN, 0);   // enable driver (active LOW)

gpio_put(MOTOR_DIR_PIN, 1);   // default forward

}

static void step_pulse_m(uint32_t pulse_width_us) {

gpio_put(MOTOR_PUL_PIN, 1); sleep_us(pulse_width_us);

gpio_put(MOTOR_PUL_PIN, 0); sleep_us(pulse_width_us);

}

// =============================================================================

// I2C helpers + IMU (ADXL345 via TCA9548A) — Core 1 only

// All I2C/GPIO operations live here; Core 0 reads only volatile caches.

// Functions are verbatim from KINETIX_test.c except for static linkage.

// =============================================================================

typedef struct { float ax, ay, az; } imu_accel_t;

static void my_i2c_init(void) {

i2c_init(I2C_PORT, I2C_BAUDRATE);

gpio_set_function(I2C_SDA_PIN, GPIO_FUNC_I2C);

gpio_set_function(I2C_SCL_PIN, GPIO_FUNC_I2C);

gpio_pull_up(I2C_SDA_PIN);

gpio_pull_up(I2C_SCL_PIN);

}

static int8_t i2c_write_bytes(uint8_t dev, uint8_t reg, uint8_t *data, size_t len) {

uint8_t buf[len + 1];

buf[0] = reg;

for (size_t i = 0; i < len; i++) buf[i + 1] = data[i];

return (int8_t)i2c_write_blocking(I2C_PORT, dev, buf, len + 1, false);

}

static int8_t i2c_read_bytes(uint8_t dev, uint8_t reg, uint8_t *data, size_t len) {

int ret = i2c_write_blocking(I2C_PORT, dev, &reg, 1, true);

if (ret < 0) return (int8_t)ret;

return (int8_t)i2c_read_blocking(I2C_PORT, dev, data, len, false);

}

static int tca_select(uint8_t ch) {

if (ch > 7) return -1;

uint8_t d = (uint8_t)(1u << ch);

return (i2c_write_blocking(I2C_PORT, TCA9548A_ADDR, &d, 1, false) == 1) ? 0 : -2;

}

static void adxl345_init(void) {

uint8_t d;

d = 0x00; i2c_write_bytes(ADXL345_ADDR, 0x2D, &d, 1);  // standby

d = 0x0A; i2c_write_bytes(ADXL345_ADDR, 0x2C, &d, 1);  // 100 Hz

d = 0x08; i2c_write_bytes(ADXL345_ADDR, 0x31, &d, 1);  // full-res ±2g

d = 0x08; i2c_write_bytes(ADXL345_ADDR, 0x2D, &d, 1);  // measure

}

static int read_accel(imu_accel_t *out) {

uint8_t raw[6];

if (i2c_read_bytes(ADXL345_ADDR, 0x32, raw, 6) < 0) {

out->ax = out->ay = out->az = 0.0f;

return -1;

}

out->ax = (float)(int16_t)((raw[1] << 8) | raw[0]) * 0.004f * 9.80665f;

out->ay = (float)(int16_t)((raw[3] << 8) | raw[2]) * 0.004f * 9.80665f;

out->az = (float)(int16_t)((raw[5] << 8) | raw[4]) * 0.004f * 9.80665f;

return 0;

}

// ── Angle caches — written by Core 1, read by Core 0 ─────────────────────────

//

// Angle computation (accelerometer only, quasi-static assumption):

//   Each IMU has Z perpendicular to its limb segment.  The long axis of the

//   segment therefore lies in the IMU XY plane.  The signed angle of the

//   acceleration vector projected onto XY (relative to the +Y axis) gives the

//   segment inclination from vertical in the sagittal plane:

//     theta = atan2(ax, ay)  [radians]

//

//   Hip flexion  = thigh angle (IMU1) − trunk angle (IMU3)

//   Knee flexion = shank angle (IMU2) − thigh angle (IMU1)

//

//   Both are positive for flexion.  Adjust sign if sensors are mounted in the

//   opposite orientation.

static volatile float hip_angle_latest  = 0.0f;   // degrees

static volatile float knee_angle_latest = 0.0f;   // degrees

#define RAD_TO_DEG  (180.0f / 3.14159265f)

// =============================================================================

// Rolling average — smooths sensor noise before values reach TICK messages.

// Window size ROLLING_N.  All instances live on Core 1; Core 0 reads only

// the already-smoothed volatile caches.

// =============================================================================

typedef struct {

float   buf[ROLLING_N];

uint8_t idx;    // next write position

uint8_t count;  // number of valid samples (0 → ROLLING_N)

float   sum;    // running sum for O(1) average

} rolling_avg_t;

// Push one sample, return the current rolling mean.

static float rolling_push(rolling_avg_t *r, float val) {

if (r->count < ROLLING_N) {

r->count++;

} else {

r->sum -= r->buf[r->idx];  // evict oldest sample

}

r->buf[r->idx] = val;

r->sum        += val;

r->idx         = (uint8_t)((r->idx + 1u) % ROLLING_N);

return r->sum / (float)r->count;

}

// One buffer per sensor channel — zero-initialised as static globals.

static rolling_avg_t hip_avg;

static rolling_avg_t knee_avg;

static rolling_avg_t pf_avg;

static void update_imu_angles(void) {

imu_accel_t a;

float th1 = 0.0f, th2 = 0.0f, th3 = 0.0f;

if (tca_select(IMU1_CH) == 0 && read_accel(&a) == 0)

th1 = atan2f(a.ax, a.ay) * RAD_TO_DEG;

if (tca_select(IMU2_CH) == 0 && read_accel(&a) == 0)

th2 = atan2f(a.ax, a.ay) * RAD_TO_DEG;

if (tca_select(IMU3_CH) == 0 && read_accel(&a) == 0)

th3 = atan2f(a.ax, a.ay) * RAD_TO_DEG;

hip_angle_latest  = rolling_push(&hip_avg,  th1 - th3);

knee_angle_latest = rolling_push(&knee_avg, th2 - th1);

}

// ── Core 1 entry point ────────────────────────────────────────────────────────

// Spins forever waiting for motor_req_pending.  All GPIO and sleep_us calls

// happen here on Core 1, completely isolated from the BLE run loop on Core 0.

void core1_motor_entry(void) {

// ── Init all sensors on Core 1 ───────────────────────────────────────

hx711_init(H0_DATA, H0_CLOCK);

my_i2c_init();

const uint8_t imu_chs[3] = { IMU1_CH, IMU2_CH, IMU3_CH };

for (int i = 0; i < 3; i++) {

tca_select(imu_chs[i]);

sleep_ms(10);

adxl345_init();

}

printf("[CORE1] Sensors initialised\n");

uint32_t step_count = 0;

while (true) {

if (motor_req_pending) {

bool     fwd = motor_req_fwd;

uint32_t ms  = motor_req_ms;

motor_req_pending = false;

motor_stop_req    = false;

stepper_init_m();

gpio_put(MOTOR_DIR_PIN, fwd ? 1 : 0);

absolute_time_t end_time = make_timeout_time_ms(ms);

while (!time_reached(end_time) && !motor_stop_req) {

step_pulse_m(MOTOR_DELAY_US);

step_count++;

// HX711: sample after every step when data pin is ready (~50 µs)

if (gpio_get(H0_DATA) == 0)

pf_latest = rolling_push(&pf_avg,

hx711_read_force_nowait(H0_DATA, H0_CLOCK, 0));

// IMU: sample every 100 steps (~500 ms) to stay well within

// the 1 Hz TICK rate without adding significant step overhead.

if (step_count % 100 == 0)

update_imu_angles();

}

motor_stop_req = false;

motor_is_done  = true;

} else {

// Idle — sample both sensors continuously.

if (gpio_get(H0_DATA) == 0)

pf_latest = rolling_push(&pf_avg,

hx711_read_force_nowait(H0_DATA, H0_CLOCK, 0));

update_imu_angles();

}

}

}

// ── Motor test poll timer (used only by the manual Fwd/Bwd test buttons) ─────

typedef struct { btstack_timer_source_t timer; } motor_poll_ctx_t;

static motor_poll_ctx_t motor_poll_ctx;

static bool             motor_poll_armed = false;

static void motor_poll_cb(btstack_timer_source_t *ts) {

if (motor_is_done) {

motor_is_done    = false;

motor_poll_armed = false;

ble_enqueue("MOTOR_DONE");

hello_ble_send();

printf("[MOTOR] Done\n");

return;   // disarm

}

btstack_run_loop_set_timer(ts, MOTOR_POLL_MS);

btstack_run_loop_add_timer(ts);

}

// =============================================================================

// Motor session state machine (Core 0 / btstack)

// Drives passive HKF / HKE exercises:

//   FWD MOTOR_DURATION ms → BWD MOTOR_DURATION ms → rep done → repeat

// TICK is sent every ~1 s during the FWD phase (Ns = 1..MOTOR_DURATION/1000).

// Rep/set counters advance only after BWD completes.

// =============================================================================

// Returns true for any passive HKF or HKE exercise — these use the motor.

static bool is_motor_exercise(const exercise_t *ex) {

return ex->mode == 'P' &&

(strstr(ex->code, "HKF") != NULL ||

strstr(ex->code, "HKE") != NULL);

}

typedef struct {

btstack_timer_source_t timer;

bool            fwd_phase;           // true = FWD, false = BWD

uint32_t        tick_sent;           // last Ns value sent as a TICK

uint32_t        total_ms;            // cumulative ms for data log

uint32_t        phase_duration_ms;   // FWD (and BWD) duration

bool            was_paused;          // true after Core 1 was stopped for pause

uint32_t        paused_remaining;    // ms left in phase when paused

absolute_time_t phase_start_abs;     // Core 0 hardware time when phase started

uint32_t        phase_pre_elapsed_ms;// ms elapsed before current Core 1 run (resume)

} motor_session_ctx_t;

static motor_session_ctx_t ms_ctx;

static bool                ms_running = false;

// Signal Core 1 to start a new FWD phase and reset per-phase counters.

// phase_ms: duration of both the FWD and BWD strokes for this exercise.

static void start_motor_rep_fwd(motor_session_ctx_t *ms, uint32_t phase_ms) {

ms->fwd_phase             = true;

ms->tick_sent             = 0;

ms->phase_duration_ms     = phase_ms;

ms->was_paused            = false;

ms->paused_remaining      = 0;

ms->phase_pre_elapsed_ms  = 0;

ms->phase_start_abs       = get_absolute_time();  // real hardware time

motor_req_fwd             = true;

motor_req_ms              = phase_ms;

motor_is_done             = false;

motor_stop_req            = false;

motor_req_pending         = true;

}

static void motor_session_poll_cb(btstack_timer_source_t *ts) {

motor_session_ctx_t *ms = (motor_session_ctx_t *)btstack_run_loop_get_timer_context(ts);

run_ctx_t           *c  = &run_ctx;

if (!ms_running) return;

// Phase guard — abort if session ended externally

if (phase != PHASE_WARMUP_RUN && phase != PHASE_SESSION_RUN) {

ms_running = false;

return;

}

if (stopped) {

motor_stop_req = true;   // ask Core 1 to abort current run

ms_running     = false;

timer_armed    = false;

phase          = PHASE_IDLE;

printf("[MOTOR SESSION] STOP\n");

return;

}

if (paused) {

if (!ms->was_paused) {

// First tick while paused — record real elapsed, stop Core 1

uint32_t real_so_far = ms->phase_pre_elapsed_ms +

(uint32_t)(absolute_time_diff_us(ms->phase_start_abs,

get_absolute_time()) / 1000);

ms->paused_remaining = (ms->phase_duration_ms > real_so_far)

? (ms->phase_duration_ms - real_so_far) : 0;

ms->was_paused  = true;

motor_stop_req  = true;

printf("[MOTOR SESSION] PAUSED — %lu ms remaining\n",

(unsigned long)ms->paused_remaining);

}

if (motor_is_done) motor_is_done = false;

btstack_run_loop_set_timer(ts, MOTOR_POLL_MS);

btstack_run_loop_add_timer(ts);

return;

}

// ── Resume after pause ────────────────────────────────────────────────

if (ms->was_paused) {

ms->was_paused            = false;

// pre_elapsed = how far into the phase we already were

ms->phase_pre_elapsed_ms  = ms->phase_duration_ms - ms->paused_remaining;

ms->phase_start_abs       = get_absolute_time();  // reset clock for new run

printf("[MOTOR SESSION] RESUMED — %lu ms remaining\n",

(unsigned long)ms->paused_remaining);

if (ms->paused_remaining > 0) {

motor_req_fwd     = ms->fwd_phase;

motor_req_ms      = ms->paused_remaining;

motor_is_done     = false;

motor_stop_req    = false;

motor_req_pending = true;

} else {

motor_is_done = true;

}

}

// ── Real elapsed in current phase (hardware clock, not callback count) ──

uint32_t real_elapsed_ms = ms->phase_pre_elapsed_ms +

(uint32_t)(absolute_time_diff_us(ms->phase_start_abs,

get_absolute_time()) / 1000);

ms->total_ms += MOTOR_POLL_MS;  // total_ms still approximate; data log only

// ── Send TICK every 1 s using real hardware time ──────────────────────

{

uint32_t Ns = real_elapsed_ms / 1000;

if (Ns > ms->tick_sent) {

ms->tick_sent = Ns;

const exercise_t *ex = &plan[c->ex_idx];

float hip  = get_hip_angle();

float knee = get_knee_angle();

float pf   = get_plantar_force();

float tf   = get_thigh_force();

float mf   = get_medial_force();

float lf   = get_lateral_force();

// Log to session data buffer (FWD phase only)

if (ms->fwd_phase && phase == PHASE_SESSION_RUN && data_count < MAX_DATA_ENTRIES) {

data_entry_t *e = &data_log[data_count++];

strncpy(e->code, ex->code, sizeof(e->code) - 1);

e->code[sizeof(e->code)-1] = '\0';

e->t_ms = ms->total_ms;

e->rep  = c->rep_idx + 1;

e->set  = c->set_idx + 1;

e->hip  = hip;  e->knee = knee;

e->pf   = pf;   e->tf   = tf;

e->mf   = mf;   e->lf   = lf;

}

char msg[128];

if (ms->fwd_phase) {

// FWD: send elapsed seconds — UI uses this for progress ring

snprintf(msg, sizeof(msg),

"TICK|%s|%us|%u|%u|%.2f|%.2f|%.2f|%.2f|%.2f|%.2f",

ex->code, (unsigned)Ns,

(unsigned)(c->rep_idx + 1),

(unsigned)(c->set_idx + 1),

hip, knee, pf, tf, mf, lf);

} else {

// BWD: send RESETTING — UI shows "RESETTING" and collapses ring

snprintf(msg, sizeof(msg),

"TICK|%s|RESETTING|%u|%u|%.2f|%.2f|%.2f|%.2f|%.2f|%.2f",

ex->code,

(unsigned)(c->rep_idx + 1),

(unsigned)(c->set_idx + 1),

hip, knee, pf, tf, mf, lf);

}

ble_enqueue(msg);

hello_ble_send();

printf("%s\n", msg);

}

}

// ── Check for motor phase completion ──────────────────────────────────

if (!motor_is_done) {

btstack_run_loop_set_timer(ts, MOTOR_POLL_MS);

btstack_run_loop_add_timer(ts);

return;

}

motor_is_done = false;

if (ms->fwd_phase) {

// FWD done — start BWD (reset stroke)

printf("[MOTOR SESSION] FWD done → BWD\n");

ms->fwd_phase             = false;

ms->tick_sent             = 0;

ms->phase_pre_elapsed_ms  = 0;

ms->phase_start_abs       = get_absolute_time();

motor_req_fwd             = false;

motor_req_ms              = ms->phase_duration_ms;

motor_is_done             = false;

motor_stop_req            = false;

motor_req_pending         = true;

btstack_run_loop_set_timer(ts, MOTOR_POLL_MS);

btstack_run_loop_add_timer(ts);

return;

}

// ── BWD done — rep complete ───────────────────────────────────────────

const exercise_t *ex = &plan[c->ex_idx];

printf("[MOTOR SESSION] Rep %u/%u Set %u/%u done\n",

(unsigned)(c->rep_idx + 1), (unsigned)ex->reps,

(unsigned)(c->set_idx + 1), (unsigned)ex->sets);

c->rep_idx++;

if (c->rep_idx < ex->reps) {

start_motor_rep_fwd(ms, ms->phase_duration_ms);

btstack_run_loop_set_timer(ts, MOTOR_POLL_MS);

btstack_run_loop_add_timer(ts);

return;

}

c->rep_idx = 0;

c->set_idx++;

if (c->set_idx < ex->sets) {

start_motor_rep_fwd(ms, ms->phase_duration_ms);

btstack_run_loop_set_timer(ts, MOTOR_POLL_MS);

btstack_run_loop_add_timer(ts);

return;

}

c->set_idx = 0;

c->ex_idx++;

if (c->ex_idx < plan_received) {

const exercise_t *next = &plan[c->ex_idx];

printf("[KINETIX] Next exercise: %s (%c)\n", next->code, next->mode);

if (is_motor_exercise(next)) {

uint32_t next_ms = (next->param > 0) ? (uint32_t)next->param * 1000u : MOTOR_DURATION;

start_motor_rep_fwd(ms, next_ms);

btstack_run_loop_set_timer(ts, MOTOR_POLL_MS);

btstack_run_loop_add_timer(ts);

} else {

// Switch back to the standard 1 s tick timer

ms_running = false;

c->tick    = 0;

btstack_run_loop_set_timer_context(&run_ctx.timer, &run_ctx);

btstack_run_loop_set_timer_handler(&run_ctx.timer, tick_callback);

btstack_run_loop_set_timer(&run_ctx.timer, 1000);

btstack_run_loop_add_timer(&run_ctx.timer);

}

return;

}

// ── All exercises complete ────────────────────────────────────────────

ms_running  = false;

timer_armed = false;

ble_enqueue("SESSION_DONE");

hello_ble_send();

const char *lbl = (phase == PHASE_WARMUP_RUN) ? "WARMUP" : "SESSION";

printf("[KINETIX] (%s) Complete — SESSION_DONE sent\n", lbl);

if (phase == PHASE_SESSION_RUN)

printf("[KINETIX] Data log: %u entries\n", (unsigned)data_count);

phase = PHASE_IDLE;

}

// =============================================================================

// HX711 load cell functions — verbatim from MOTORWORKS.c

// Called exclusively from Core 1. Core 0 reads only the pf_latest cache.

// hx711_wait_ready is never called from Core 0 — no CYW43 watchdog risk.

// =============================================================================

static void hx711_init(uint8_t data_pin, uint8_t clock_pin) {

gpio_init(data_pin);

gpio_set_dir(data_pin, GPIO_IN);

gpio_init(clock_pin);

gpio_set_dir(clock_pin, GPIO_OUT);

gpio_put(clock_pin, 0);

}

static void hx711_wait_ready(uint8_t data_pin) {

while (gpio_get(data_pin) == 1) {

tight_loop_contents();

}

}

static int32_t hx711_read_raw(uint8_t data_pin, uint8_t clock_pin) {

hx711_wait_ready(data_pin);

int32_t data = 0;

for (int i = 0; i < 24; i++) {

gpio_put(clock_pin, 1); sleep_us(1);

data = (data << 1) | gpio_get(data_pin);

gpio_put(clock_pin, 0); sleep_us(1);

}

gpio_put(clock_pin, 1); sleep_us(1);

gpio_put(clock_pin, 0); sleep_us(1);

if (data & 0x800000) data |= ~0xFFFFFF;

return data;

}

static float hx711_get_force(uint8_t data_pin, uint8_t clock_pin,

uint8_t unit, int samples) {

int64_t sum = 0;

for (int s = 0; s < samples; s++) sum += hx711_read_raw(data_pin, clock_pin);

int32_t avg   = (int32_t)(sum / samples);

float   force = (float)(avg - hx711_offset[unit]) / hx711_scale[unit];

if (force < 0.0f) force = 0.0f;

return force;

}

// =============================================================================

// Sensor placeholder functions

// ─────────────────────────────────────────────────────────────────────────────

// Each returns 0.0f until real hardware reads are implemented.

// Replace the body of each function with your IMU / HX711 read call.

// The rest of the firmware does not need to change.

// =============================================================================

static float get_hip_angle(void)  { return (float)hip_angle_latest; }

static float get_knee_angle(void) { return (float)knee_angle_latest; }

// Read 24 bits WITHOUT a blocking wait (data pin must already be LOW).

// Called only from Core 1 — gpio_put / sleep_us are safe here.

static float hx711_read_force_nowait(uint8_t data_pin, uint8_t clock_pin,

uint8_t unit) {

int32_t raw = 0;

for (int i = 0; i < 24; i++) {

gpio_put(clock_pin, 1); sleep_us(1);

raw = (raw << 1) | gpio_get(data_pin);

gpio_put(clock_pin, 0); sleep_us(1);

}

// 25th pulse sets Channel A gain 128 for next conversion

gpio_put(clock_pin, 1); sleep_us(1);

gpio_put(clock_pin, 0); sleep_us(1);

if (raw & 0x800000) raw |= ~0xFFFFFF;

float force = (float)(raw - hx711_offset[unit]) / hx711_scale[unit];

return (force < 0.0f) ? 0.0f : force;

}

// Core 0 NEVER calls gpio_put/get for HX711 — returns the Core 1 cache only.

static float get_plantar_force(void) { return (float)pf_latest; }

static float get_thigh_force(void)   { return 0.0f; }  // TODO: HX711 thigh sensor

static float get_medial_force(void)  { return 0.0f; }  // TODO: HX711 medial sensor

static float get_lateral_force(void) { return 0.0f; }  // TODO: HX711 lateral sensor

// =============================================================================

// Report flush timer — sends one DATA| line every FLUSH_INTERVAL_MS

// =============================================================================

static void flush_timer_callback(btstack_timer_source_t *ts) {

flush_ctx_t *fc = (flush_ctx_t *)btstack_run_loop_get_timer_context(ts);

if (fc->idx >= data_count) {

// All entries sent

ble_enqueue("REPORT_DONE");

hello_ble_send();

printf("[KINETIX] Report flush complete — %u entries sent\n",

(unsigned)data_count);

data_count  = 0;   // clear log for next session

timer_armed = false;

phase       = PHASE_IDLE;

return;

}

const data_entry_t *e = &data_log[fc->idx++];

char msg[128];

snprintf(msg, sizeof(msg),

"DATA|%s|%lu|%u|%u|%.2f|%.2f|%.2f|%.2f|%.2f|%.2f",

e->code,

(unsigned long)e->t_ms,

(unsigned)e->rep,

(unsigned)e->set,

e->hip, e->knee,

e->pf,  e->tf,

e->mf,  e->lf);

ble_enqueue(msg);

hello_ble_send();

btstack_run_loop_set_timer(ts, FLUSH_INTERVAL_MS);

btstack_run_loop_add_timer(ts);

}

static void start_flush(void) {

if (data_count == 0) {

ble_enqueue("REPORT_DONE");

hello_ble_send();

printf("[KINETIX] No data to flush\n");

return;

}

printf("[KINETIX] Starting flush — %u entries at %d ms/entry\n",

(unsigned)data_count, FLUSH_INTERVAL_MS);

phase         = PHASE_FLUSH_RUN;

timer_armed   = true;

flush_ctx.idx = 0;

btstack_run_loop_set_timer_context(&flush_ctx.timer, &flush_ctx);

btstack_run_loop_set_timer_handler(&flush_ctx.timer, flush_timer_callback);

btstack_run_loop_set_timer(&flush_ctx.timer, FLUSH_INTERVAL_MS);

btstack_run_loop_add_timer(&flush_ctx.timer);

}

// =============================================================================

// Tick timer — fires every 1 000 ms during WARMUP_RUN or SESSION_RUN

// =============================================================================

static void tick_callback(btstack_timer_source_t *ts) {

run_ctx_t *c = (run_ctx_t *)btstack_run_loop_get_timer_context(ts);

// ── Phase guard ────────────────────────────────────────────────────────────

// Reject spurious callbacks that arrive after the phase has already moved

// to IDLE (e.g. natural session completion, mid-session REPORT, or STOP).

// Without this guard a late-firing callback reads plan[ex_idx] where

// ex_idx == plan_received — one past the end of the array — which is

// undefined behaviour and causes the last exercise to appear to repeat.

if (phase != PHASE_WARMUP_RUN && phase != PHASE_SESSION_RUN) {

timer_armed = false;

return;

}

if (stopped) {

printf("[KINETIX] Aborted by STOP\n");

timer_armed = false;

phase       = PHASE_IDLE;

return;

}

if (paused) {

btstack_run_loop_set_timer(ts, 1000);

btstack_run_loop_add_timer(ts);

return;

}

c->tick++;

c->total_tick++;

const exercise_t *ex = &plan[c->ex_idx];

// ── Read sensors once per tick ─────────────────────────────────────────

float hip  = get_hip_angle();

float knee = get_knee_angle();

float pf   = get_plantar_force();

float tf   = get_thigh_force();

float mf   = get_medial_force();

float lf   = get_lateral_force();

// ── Log to data buffer during SESSION_RUN only ─────────────────────────

if (phase == PHASE_SESSION_RUN && data_count < MAX_DATA_ENTRIES) {

data_entry_t *entry = &data_log[data_count++];

strncpy(entry->code, ex->code, sizeof(entry->code) - 1);

entry->code[sizeof(entry->code) - 1] = '\0';

entry->t_ms  = (uint32_t)(c->total_tick * 1000u);

entry->rep   = c->rep_idx + 1;

entry->set   = c->set_idx + 1;

entry->hip   = hip;

entry->knee  = knee;

entry->pf    = pf;

entry->tf    = tf;

entry->mf    = mf;

entry->lf    = lf;

}

// ── Build progress field ───────────────────────────────────────────────

char progress[12];

bool rep_done;

if (ex->mode == 'A') {

uint8_t pct = (uint8_t)((c->tick * 100u) / ACTIVE_REP_SEC);

if (pct > 100) pct = 100;

snprintf(progress, sizeof(progress), "%u", (unsigned)pct);

rep_done = (c->tick >= ACTIVE_REP_SEC);

} else {

snprintf(progress, sizeof(progress), "%us", (unsigned)c->tick);

rep_done = (c->tick >= ex->param);

}

// ── Emit TICK|CODE|progress|rep|set|hip|knee|pf|tf|mf|lf ──────────────

char msg[128];

snprintf(msg, sizeof(msg),

"TICK|%s|%s|%u|%u|%.2f|%.2f|%.2f|%.2f|%.2f|%.2f",

ex->code,

progress,

(unsigned)(c->rep_idx + 1),

(unsigned)(c->set_idx + 1),

hip, knee, pf, tf, mf, lf);

ble_enqueue(msg);

hello_ble_send();

printf("%s\n", msg);

// ── Advance counters on rep completion ─────────────────────────────────

if (!rep_done) {

btstack_run_loop_set_timer(ts, 1000);

btstack_run_loop_add_timer(ts);

return;

}

c->tick = 0;

c->rep_idx++;

if (c->rep_idx < ex->reps) {

btstack_run_loop_set_timer(ts, 1000);

btstack_run_loop_add_timer(ts);

return;

}

c->rep_idx = 0;

c->set_idx++;

if (c->set_idx < ex->sets) {

btstack_run_loop_set_timer(ts, 1000);

btstack_run_loop_add_timer(ts);

return;

}

c->set_idx = 0;

c->ex_idx++;

if (c->ex_idx < plan_received) {

const char *label = (phase == PHASE_WARMUP_RUN) ? "WARMUP" : "SESSION";

printf("[KINETIX] (%s) Exercise %u/%u — %s (%c)\n",

label,

(unsigned)(c->ex_idx + 1), (unsigned)plan_received,

plan[c->ex_idx].code, plan[c->ex_idx].mode);

btstack_run_loop_set_timer(ts, 1000);

btstack_run_loop_add_timer(ts);

return;

}

// ── Phase complete ─────────────────────────────────────────────────────

ble_enqueue("SESSION_DONE");

hello_ble_send();

const char *label = (phase == PHASE_WARMUP_RUN) ? "WARMUP" : "SESSION";

printf("[KINETIX] (%s) Complete — SESSION_DONE sent\n", label);

if (phase == PHASE_SESSION_RUN)

printf("[KINETIX] Data log: %u entries — send REPORT to flush\n",

(unsigned)data_count);

timer_armed = false;

phase       = PHASE_IDLE;

// Explicit return — belt-and-suspenders: ensures the timer is never

// re-armed by any code that might be added below this point in future.

return;

}

// ── Arm the tick timer ────────────────────────────────────────────────────────

static void arm_timer(void) {

run_ctx.ex_idx     = 0;

run_ctx.set_idx    = 0;

run_ctx.rep_idx    = 0;

run_ctx.tick       = 0;

run_ctx.total_tick = 0;

const char *label = (phase == PHASE_WARMUP_RUN) ? "WARMUP" : "SESSION";

printf("[KINETIX] (%s) Starting — %u exercise(s)\n", label,

(unsigned)plan_received);

for (uint8_t i = 0; i < plan_received; i++)

printf("  [%u] code=%-20s sets=%u reps=%u mode=%c param=%u\n",

(unsigned)(i + 1), plan[i].code,

(unsigned)plan[i].sets, (unsigned)plan[i].reps,

plan[i].mode, (unsigned)plan[i].param);

if (plan_received > 0 && is_motor_exercise(&plan[0])) {

// Motor-driven session: use the 50 ms poll callback

ms_ctx.fwd_phase             = false;  // start_motor_rep_fwd sets this

ms_ctx.tick_sent             = 0;

ms_ctx.total_ms              = 0;

ms_ctx.phase_pre_elapsed_ms  = 0;

ms_ctx.phase_start_abs       = get_absolute_time();

ms_running        = true;

uint32_t phase_ms = (plan[0].param > 0)

? (uint32_t)plan[0].param * 1000u

: MOTOR_DURATION;  // fallback: 17 s

start_motor_rep_fwd(&ms_ctx, phase_ms);

btstack_run_loop_set_timer_context(&ms_ctx.timer, &ms_ctx);

btstack_run_loop_set_timer_handler(&ms_ctx.timer, motor_session_poll_cb);

btstack_run_loop_set_timer(&ms_ctx.timer, MOTOR_POLL_MS);

btstack_run_loop_add_timer(&ms_ctx.timer);

} else {

// Standard 1 s tick-based session

btstack_run_loop_set_timer_context(&run_ctx.timer, &run_ctx);

btstack_run_loop_set_timer_handler(&run_ctx.timer, tick_callback);

btstack_run_loop_set_timer(&run_ctx.timer, 1000);

btstack_run_loop_add_timer(&run_ctx.timer);

}

timer_armed = true;

}

// ── Cancel whichever timer is currently armed ─────────────────────────────────

static void cancel_timer(void) {

if (!timer_armed) return;

if (phase == PHASE_FLUSH_RUN) {

btstack_run_loop_remove_timer(&flush_ctx.timer);

} else if (ms_running) {

btstack_run_loop_remove_timer(&ms_ctx.timer);

ms_running     = false;

motor_stop_req = true;   // abort any in-progress motor run on Core 1

} else {

btstack_run_loop_remove_timer(&run_ctx.timer);

}

timer_armed = false;

printf("[KINETIX] Timer cancelled\n");

}

// ── Parse one EX| line into plan[plan_received] ───────────────────────────────

static int parse_ex(const char *line) {

char buf[80];

strncpy(buf, line, sizeof(buf) - 1);

buf[sizeof(buf) - 1] = '\0';

char *tok = strtok(buf, "|");                // "EX"

tok = strtok(NULL, "|"); if (!tok) return 0; // code

strncpy(plan[plan_received].code, tok,

sizeof(plan[plan_received].code) - 1);

plan[plan_received].code[sizeof(plan[plan_received].code) - 1] = '\0';

tok = strtok(NULL, "|"); if (!tok) return 0; // sets

plan[plan_received].sets = (uint8_t)atoi(tok);

tok = strtok(NULL, "|"); if (!tok) return 0; // reps

plan[plan_received].reps = (uint8_t)atoi(tok);

tok = strtok(NULL, "|"); if (!tok) return 0; // mode

plan[plan_received].mode = tok[0];

tok = strtok(NULL, "|"); if (!tok) return 0; // param

plan[plan_received].param = (uint8_t)atoi(tok);

if (plan[plan_received].mode == 'P' && plan[plan_received].param == 0)

plan[plan_received].param = 5;

return 1;

}

// =============================================================================

// Diagnostic burst timer — fires every DIAG_INTERVAL_MS on the btstack run loop

// Sends DIAG_PACKETS packets at 5 Hz, then DIAG_DONE, then disarms itself.

// =============================================================================

static void diag_timer_callback(btstack_timer_source_t *ts) {

diag_ctx_t *dc = (diag_ctx_t *)btstack_run_loop_get_timer_context(ts);

dc->seq++;

char msg[24];

snprintf(msg, sizeof(msg), "DIAG|%u", (unsigned)dc->seq);

ble_enqueue(msg);

hello_ble_send();

printf("[DIAG] %s\n", msg);

if (dc->seq >= DIAG_PACKETS) {

ble_enqueue("DIAG_DONE");

hello_ble_send();

printf("[DIAG] Burst complete — %u packets sent\n", (unsigned)DIAG_PACKETS);

diag_running = false;

return;   // not re-armed — burst finished

}

btstack_run_loop_set_timer(ts, DIAG_INTERVAL_MS);

btstack_run_loop_add_timer(ts);

}

// =============================================================================

// BLE receive callback

// =============================================================================

void on_ble_receive(const char *line) {

printf("[RX] %s\n", line);

// ── PING — keepalive, discard silently ────────────────────────────────────

if (strcmp(line, "PING") == 0) return;

// ── LATPING|<seq> — immediate round-trip latency probe ────────────────────

// Accepted in any phase so it never disturbs an ongoing session.

if (strncmp(line, "LATPING|", 8) == 0) {

char pong[32];

snprintf(pong, sizeof(pong), "LATPONG|%s", line + 8);

ble_enqueue(pong);

hello_ble_send();

return;

}

// ── DIAGTEST — trigger a 5 Hz burst of DIAG_PACKETS packets ───────────────

// Only accepted in PHASE_IDLE to prevent interference with sessions.

if (strcmp(line, "DIAGTEST") == 0) {

if (phase != PHASE_IDLE || diag_running) {

ble_enqueue("DIAG_BUSY");

hello_ble_send();

printf("[DIAG] DIAGTEST rejected — phase=%d running=%d\n",

(int)phase, (int)diag_running);

return;

}

diag_running = true;

diag_ctx.seq = 0;

btstack_run_loop_set_timer_context(&diag_ctx.timer, &diag_ctx);

btstack_run_loop_set_timer_handler(&diag_ctx.timer, diag_timer_callback);

btstack_run_loop_set_timer(&diag_ctx.timer, DIAG_INTERVAL_MS);

btstack_run_loop_add_timer(&diag_ctx.timer);

printf("[DIAG] DIAGTEST started — %u packets at %d ms intervals\n",

(unsigned)DIAG_PACKETS, DIAG_INTERVAL_MS);

return;

}

// ── STOP ─────────────────────────────────────────────────────────────────

// Preserves data_count so REPORT still works after a manual end-session.

if (strcmp(line, "STOP") == 0) {

cancel_timer();

stopped = true;

paused  = false;

phase   = PHASE_IDLE;

ble_enqueue("STOPPED");

hello_ble_send();

printf("[KINETIX] STOP — data log holds %u entries\n",

(unsigned)data_count);

return;

}

// ── PAUSE ─────────────────────────────────────────────────────────────────

if (strcmp(line, "PAUSE") == 0) {

paused = true;

ble_enqueue("PAUSED");

hello_ble_send();

return;

}

// ── RESUME ────────────────────────────────────────────────────────────────

if (strcmp(line, "RESUME") == 0) {

paused = false;

ble_enqueue("RESUMED");

hello_ble_send();

return;

}

// ── REPORT ────────────────────────────────────────────────────────────────

// Accepted in two situations:

//   (a) PHASE_IDLE  — session completed naturally or was already stopped.

//   (b) PHASE_SESSION_RUN — clinician pressed "End Session" early.

//       The tick timer is cancelled immediately, the data log is preserved,

//       and the flush begins.  This is the fix for the REPORT_BUSY symptom

//       that occurs when the dashboard requests data before STOP is received.

if (strcmp(line, "REPORT") == 0) {

if (phase == PHASE_SESSION_RUN) {

// Mid-session early exit: stop the tick timer, keep data intact.

cancel_timer();

stopped = true;

paused  = false;

phase   = PHASE_IDLE;

printf("[KINETIX] REPORT mid-session — cancelled tick, flushing %u entries\n",

(unsigned)data_count);

start_flush();

return;

}

if (phase != PHASE_IDLE) {

// Any other running phase (warmup, flush already in progress, etc.)

ble_enqueue("REPORT_BUSY");

hello_ble_send();

printf("[KINETIX] REPORT rejected — phase=%d\n", (int)phase);

return;

}

start_flush();

return;

}

// ── WARMUP|N ──────────────────────────────────────────────────────────────

if (strncmp(line, "WARMUP|", 7) == 0) {

cancel_timer();

plan_expected = (uint8_t)atoi(line + 7);

if (plan_expected > MAX_EXERCISES) plan_expected = MAX_EXERCISES;

plan_received = 0;

stopped       = false;

paused        = false;

phase         = PHASE_WARMUP_RX;

printf("[KINETIX] WARMUP — expecting %u exercise(s)\n",

(unsigned)plan_expected);

return;

}

// ── START|N ───────────────────────────────────────────────────────────────

if (strncmp(line, "START|", 6) == 0) {

cancel_timer();

data_count    = 0;   // clear log for new session

plan_expected = (uint8_t)atoi(line + 6);

if (plan_expected > MAX_EXERCISES) plan_expected = MAX_EXERCISES;

plan_received = 0;

stopped       = false;

paused        = false;

phase         = PHASE_SESSION_RX;

printf("[KINETIX] SESSION — expecting %u exercise(s)\n",

(unsigned)plan_expected);

return;

}

// ── EX| ───────────────────────────────────────────────────────────────────

if (strncmp(line, "EX|", 3) == 0 &&

(phase == PHASE_WARMUP_RX || phase == PHASE_SESSION_RX) &&

plan_expected > 0)

{

if (plan_received >= plan_expected) return;

if (!parse_ex(line)) {

printf("[KINETIX] EX parse error: %s\n", line);

return;

}

printf("[KINETIX] EX[%u] code=%-20s sets=%u reps=%u mode=%c param=%u\n",

(unsigned)plan_received,

plan[plan_received].code,

(unsigned)plan[plan_received].sets,

(unsigned)plan[plan_received].reps,

plan[plan_received].mode,

(unsigned)plan[plan_received].param);

plan_received++;

if (plan_received >= plan_expected) {

if (phase == PHASE_WARMUP_RX) {

phase = PHASE_WARMUP_RUN;

ble_enqueue("THE DEVICE HAS RECEIVED THE WARMUP PLAN");

} else {

phase = PHASE_SESSION_RUN;

ble_enqueue("THE DEVICE HAS RECEIVED THE TREATMENT PLAN");

}

hello_ble_send();

printf("[KINETIX] Plan complete — arming timer\n");

arm_timer();

}

return;

}

// ── MOTOR_FWD|<ms> / MOTOR_BWD|<ms> ─────────────────────────────────────

// Signals Core 1 to run the motor; Core 0 (BLE) never blocks.

if (strncmp(line, "MOTOR_FWD|", 10) == 0 || strncmp(line, "MOTOR_BWD|", 10) == 0) {

if (phase != PHASE_IDLE || motor_req_pending) {

ble_enqueue("MOTOR_BUSY");

hello_ble_send();

return;

}

bool fwd = (strncmp(line, "MOTOR_FWD|", 10) == 0);

uint32_t ms = (uint32_t)atoi(line + 10);

if (ms == 0 || ms > 10000) ms = 3000;

motor_req_fwd     = fwd;

motor_req_ms      = ms;

motor_is_done     = false;

motor_req_pending = true;   // Core 1 picks this up

if (!motor_poll_armed) {

motor_poll_armed = true;

btstack_run_loop_set_timer_context(&motor_poll_ctx.timer, &motor_poll_ctx);

btstack_run_loop_set_timer_handler(&motor_poll_ctx.timer, motor_poll_cb);

btstack_run_loop_set_timer(&motor_poll_ctx.timer, MOTOR_POLL_MS);

btstack_run_loop_add_timer(&motor_poll_ctx.timer);

}

printf("[MOTOR] %s %lu ms — Core 1 started\n", fwd ? "FWD" : "BWD", (unsigned long)ms);

return;

}

// ── SREAD — one-shot sensor snapshot for diagnostics (PHASE_IDLE only) ───

// Phone sends SREAD at ~2 Hz; Pico replies SENSOR|hip|knee|pf.

// Only accepted when idle so it cannot interfere with a running session.

if (strcmp(line, "SREAD") == 0) {

if (phase != PHASE_IDLE) {

ble_enqueue("SENSOR_BUSY");

hello_ble_send();

return;

}

char smsg[64];

snprintf(smsg, sizeof(smsg), "SENSOR|%.2f|%.2f|%.2f",

(float)hip_angle_latest,

(float)knee_angle_latest,

(float)pf_latest);

ble_enqueue(smsg);

hello_ble_send();

return;

}

printf("[RX] Unrecognised (phase=%d): %s\n", (int)phase, line);

}

// =============================================================================

// main

// =============================================================================

int main(void) {

stdio_init_all();

multicore_launch_core1(core1_motor_entry);   // Core 1 handles all motor GPIO

sleep_ms(5000);

printf("KINETIX BLE — waiting for connection...\n");

printf("  WARMUP|N  → warmup phase\n");

printf("  START|N   → treatment session\n");

printf("  REPORT    → flush logged data\n");

ble_hello_init();

}